Title: Numerical Simulation and Performance Analysis of Microwave Transmission Lines and Two-Port Networks Using Scattering Parameters

AIM

To model a coaxial microwave transmission line using MATLAB, calculate its characteristic impedance and propagation constant, analyze the scattering parameters S11, S21, S12, and S22, and evaluate return loss, insertion loss, and transmission efficiency over a specified frequency range.

Program code:

clc;

clear;

close all;

%% 1. Coaxial Transmission Line Parameters

f = linspace(1e9, 10e9, 1001);  % Frequency: 1 to 10 GHz

w = 2*pi*f;

a = 0.5e-3;        % Inner conductor radius (m)

b = 1.5e-3;        % Inner radius of outer conductor (m)

len = 0.1;         % Length of cable (m)

er = 2.1;          % Relative permittivity

mur = 1;           % Relative permeability

sigma = 5.8e7;     % Conductivity (S/m)

tand = 0.0004;     % Dielectric loss tangent

Zref = 50;         % Reference impedance (ohms)

mu0 = 4*pi*1e-7;

eps0 = 8.854e-12;

mu = mu0*mur;

eps = eps0*er;

%% 2. Calculate Transmission Line Parameters

% Per-unit-length inductance and capacitance

L = (mu/(2*pi))*log(b/a);

C = (2*pi*eps)/log(b/a);

% Frequency-dependent conductor resistance

Rs = sqrt(pi*f*mu0/sigma);

R = Rs/(2*pi*a) + Rs/(2*pi*b);

% Dielectric conductance

G = w*C*tand;

% Propagation constant

gamma = sqrt((R + 1i*w*L).*(G + 1i*w*C));

alpha = real(gamma);       % Attenuation constant (Np/m)

beta = imag(gamma);        % Phase constant (rad/m)

% Characteristic impedance

Zc = sqrt((R + 1i*w*L)./(G + 1i*w*C));

%% 3. Calculate ABCD Parameters

gl = gamma*len;

A = cosh(gl);

B = Zc.*sinh(gl);

Cc = sinh(gl)./Zc;

D = cosh(gl);

%% 4. Calculate S-Parameters

den = A + B/Zref + Cc*Zref + D;

S11 = (A + B/Zref - Cc*Zref - D)./den;

S21 = 2./den;

S12 = 2*(A.*D - B.*Cc)./den;

S22 = (-A + B/Zref - Cc*Zref + D)./den;

%% 5. Return Loss, Insertion Loss and Efficiency

ReturnLoss = -20*log10(max(abs(S11),1e-15));

InsertionLoss = -20*log10(max(abs(S21),1e-15));

TransmissionEfficiency = abs(S21).^2*100;

%% 6. Display Calculated Parameters

fprintf('\n--- Coaxial Transmission Line Results ---\n');

fprintf('Frequency Range: 1 GHz to 10 GHz\n');

fprintf('Length: %.3f m\n',len);

fprintf('Inner Radius: %.3f mm\n',a*1e3);

fprintf('Outer Conductor Inner Radius: %.3f mm\n',b*1e3);

fprintf('L = %.4e H/m\n',L);

fprintf('C = %.4e F/m\n',C);

fprintf('Characteristic Impedance at 5 GHz = %.3f ohms\n', ...

real(interp1(f, Zc, 5e9)));

fprintf('Attenuation Constant at 5 GHz = %.4f Np/m\n', ...

interp1(f, alpha, 5e9));

fprintf('Phase Constant at 5 GHz = %.4f rad/m\n', ...

interp1(f, beta, 5e9));

fprintf('Return Loss at 5 GHz = %.3f dB\n', ...

interp1(f, ReturnLoss, 5e9));

fprintf('Insertion Loss at 5 GHz = %.3f dB\n', ...

interp1(f, InsertionLoss, 5e9));

fprintf('Transmission Efficiency at 5 GHz = %.3f %%\n', ...

interp1(f, TransmissionEfficiency, 5e9));

%% 7. Plot S-Parameters

figure('Name','S-Parameter Analysis');

plot(f/1e9,20*log10(max(abs(S11),1e-15)),'LineWidth',1.5);

hold on;

plot(f/1e9,20*log10(max(abs(S21),1e-15)),'LineWidth',1.5);

plot(f/1e9,20*log10(max(abs(S12),1e-15)),'--','LineWidth',1.5);

plot(f/1e9,20*log10(max(abs(S22),1e-15)),'--','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('S-Parameters of Coaxial Transmission Line');

legend('S11','S21','S12','S22','Location','best');

%% 8. Plot Return Loss and Insertion Loss

figure('Name','Loss Analysis');

plot(f/1e9,ReturnLoss,'LineWidth',1.5);

hold on;

plot(f/1e9,InsertionLoss,'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Loss (dB)');

title('Return Loss and Insertion Loss');

legend('Return Loss','Insertion Loss','Location','best');

%% 9. Plot Transmission Efficiency

figure('Name','Transmission Efficiency');

plot(f/1e9,TransmissionEfficiency,'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Transmission Efficiency (%)');

title('Transmission Efficiency vs Frequency');

%% 10. Plot Characteristic Impedance

figure('Name','Characteristic Impedance');

plot(f/1e9,real(Zc),'LineWidth',1.5);

hold on;

plot(f/1e9,imag(Zc),'--','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Impedance (Ohms)');

title('Characteristic Impedance vs Frequency');

legend('Real Part','Imaginary Part','Location','best');

%% 11. Plot Propagation Constants

figure('Name','Propagation Constants');

plot(f/1e9,alpha,'LineWidth',1.5);

hold on;

plot(f/1e9,beta,'--','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Propagation Constant');

title('Propagation Constants vs Frequency');

legend('Attenuation (Np/m)','Phase (rad/m)', 'Location','best');

Expected Output:

| Parameter | Expected Output |
| --- | --- |
| Impedance (Zo) | 5.5 GHz |
| Propagation constant | 50.1 |
| S11 and S22 | 0.012 and 0.012 |
| S21 and S12 | 0.988 and 0.998 |
| Return Loss | 38.4dB |
| Insertion Loss | 0.105dB |
| Transmission Efficiency | 97.6% |

Result:

The microwave transmission line was modeled using MATLAB. The characteristic impedance, propagation constant and S-parameters were calculated, and return loss, insertion loss and transmission efficiency were evaluated over the selected frequency range.

Output:

OBJECTIVE 1: COAXIAL LINE MODEL

Length = 0.10 m

Inductance = 2.1972e-07 H/m

Capacitance = 1.0634e-10 F/m

OBJECTIVE 2: TRANSMISSION PARAMETERS

Frequency = 5.50 GHz

Characteristic Impedance = 45.456 + j-0.015 Ohms

Attenuation Constant = 0.1237 Np/m

Phase Constant = 167.0425 rad/m

OBJECTIVE 3: S-PARAMETERS AT 5.50 GHz

S11 = -0.0665 + j-0.0424

S21 = -0.5334 + j0.8276

S12 = -0.5334 + j0.8276

S22 = -0.0665 + j-0.0424

OBJECTIVE 4: PERFORMANCE ANALYSIS

Return Loss = 22.061 dB

Insertion Loss = 0.135 dB

Transmission Efficiency = 96.941 %

Plots: