Title: Simulation-Based Investigation of Microwave Semiconductor Devices and Frequency-Conversion Characteristics of a Microwave Mixer

Aim

To simulate and analyze the operating characteristics of microwave semiconductor devices and investigate the frequency-conversion characteristics of a microwave mixer using MATLAB.

Matlab Program:

clc;

clear;

close all;

%% Objective 1: Model microwave semiconductor devices

V = 0:0.1:10;

% Simplified Gunn diode I-V model

I_gunn = 0.02*V + 0.8*V.*exp(-((V-4).^2)/2);

% Simplified Schottky diode I-V model

Is = 1e-9;

n = 1.5;

Vt = 0.026;

I_schottky = Is*(exp(V/(n*Vt))-1);

I_schottky(I_schottky > 1) = NaN;

figure;

subplot(2,2,1);

plot(V,I_gunn,'b','LineWidth',1.5);

grid on;

xlabel('Voltage (V)');

ylabel('Current (A)');

title('Gunn Diode I-V Characteristics');

%% Objective 2: Analyze device characteristics

% Illustrative frequency-dependent response

f = linspace(1,10,500);  % Frequency in GHz

f0 = 5;                  % Reference frequency in GHz

response = 1./sqrt(1+((f-f0)/2).^2);

subplot(2,2,2);

plot(f,response,'r','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Normalized Response');

title('Frequency-Dependent Device Response');

%% Objective 3: Simulate microwave mixer frequency conversion

fs = 200e6;

t = 0:1/fs:2e-6;

fRF = 10e6;

fLO = 8e6;

RF = cos(2*pi*fRF*t);

LO = cos(2*pi*fLO*t);

% Mixer output

mixer_output = RF.*LO;

subplot(2,2,3);

plot(t*1e6,mixer_output,'k','LineWidth',1.2);

grid on;

xlabel('Time (\mus)');

ylabel('Amplitude');

title('Mixer Output Signal');

% Frequency spectrum

N = length(mixer_output);

Y = fft(mixer_output);

P2 = abs(Y/N);

P1 = P2(1:floor(N/2)+1);

P1(2:end-1) = 2*P1(2:end-1);

freq = fs*(0:floor(N/2))/N;

figure;

plot(freq/1e6,P1,'m','LineWidth',1.5);

grid on;

xlabel('Frequency (MHz)');

ylabel('Magnitude');

title('Mixer Output Frequency Spectrum');

xlim([0 25]);

%% Objective 4: Evaluate conversion characteristics

fIF = abs(fRF-fLO);

fSUM = fRF+fLO;

% Illustrative conversion-loss calculation

Pin_dBm = 0;

Pout_dBm = -6;

ConversionLoss = Pin_dBm-Pout_dBm;

fprintf('\nMICROWAVE MIXER SIMULATION RESULTS\n');

fprintf('---------------------------------\n');

fprintf('RF Frequency          = %.2f MHz\n',fRF/1e6);

fprintf('LO Frequency          = %.2f MHz\n',fLO/1e6);

fprintf('Intermediate Frequency= %.2f MHz\n',fIF/1e6);

fprintf('Sum Frequency         = %.2f MHz\n',fSUM/1e6);

fprintf('Conversion Loss       = %.2f dB\n',ConversionLoss);

% Frequency components

figure;

bar([fRF fLO fIF fSUM]/1e6,[1 1 0.5 0.5]);

grid on;

xlabel('Frequency (MHz)');

ylabel('Relative Amplitude');

title('Identified Frequency Components');

xticklabels({'RF','LO','IF','Sum'});

Expected Output:

| Parameter | Expected output |
| --- | --- |
| Gunn diode peak current | 0.8A |
| Reference Frequency | 5GHz |
| RF input frequency | 10MHz |
| Local oscillator frequency | 8MHz |
| Intermediate frequency | 2MHz |
| Sum frequency | 18MHz |
| Conversion loss | 6dB |

Output:

MICROWAVE MIXER SIMULATION RESULTS

---------------------------------

RF Frequency          = 10.00 MHz

LO Frequency          = 8.00 MHz

Intermediate Frequency= 2.00 MHz

Sum Frequency         = 18.00 MHz

Conversion Loss       = 6.00 dB

Plots: