Title:Computer-Aided Design and Optimization of a Microwave Impedance-Matching Network and Frequency-Selective Filter

Aim

To design and optimize a microwave impedance-matching network and frequency-selective filter using MATLAB, and evaluate their performance using simulated frequency responses.

Matlab Program:

clc;

clear;

close all;

%% Design Specifications

Z0 = 50;                    % Characteristic impedance (ohms)

ZL = 30 + 1j*20;            % Complex load impedance (ohms)

f0 = 2.4e9;                 % Design frequency (Hz)

f = linspace(1e9,4e9,1001); % Frequency range (Hz)

%% Objective 1: Impedance Matching Network

% Calculate load reflection coefficient

GammaL = (ZL-Z0)/(ZL+Z0);

RL_initial = -20*log10(abs(GammaL));

% Ideal conjugate-match transformation at design frequency

Zmatch = conj(ZL);

Gamma_matched = (Zmatch-ZL)/(Zmatch+ZL);

RL_matched = -20*log10(max(abs(Gamma_matched),eps));

fprintf('INITIAL LOAD IMPEDANCE = %.2f + j%.2f ohms\n',real(ZL),imag(ZL));

fprintf('INITIAL RETURN LOSS = %.2f dB\n',RL_initial);

fprintf('IDEAL MATCHED RETURN LOSS = %.2f dB\n',RL_matched);

%% Objective 2: Matching Network Optimization

% Illustrative frequency-dependent matching response

x = (f-f0)/f0;

Gamma = 0.03 + 1j*0.08*x;

Gamma = min(abs(Gamma),0.99).*exp(1j*angle(Gamma));

% Return loss and reflected power

ReturnLoss = -20*log10(max(abs(Gamma),eps));

ReflectedPower = abs(Gamma).^2;

% Find minimum-reflection operating frequency

[~,idx] = min(abs(Gamma));

f_opt = f(idx);

figure;

subplot(2,1,1);

plot(f/1e9,ReturnLoss,'b','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Impedance Matching Network');

subplot(2,1,2);

plot(f/1e9,ReflectedPower,'r','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Reflected Power Ratio');

title('Reflection Characteristics');

fprintf('OPTIMUM FREQUENCY = %.3f GHz\n',f_opt/1e9);

fprintf('RETURN LOSS AT OPTIMUM = %.2f dB\n',ReturnLoss(idx));

%% Objective 3: Microwave Bandpass Filter

% Second-order Butterworth-like bandpass response

fL = 2.0e9;                 % Lower cutoff frequency

fH = 2.8e9;                 % Upper cutoff frequency

fCenter = sqrt(fL*fH);

BW = fH-fL;

% Approximate bandpass magnitude response

u = (f.^2-fCenter^2)./(f*BW);

H = 1./sqrt(1+u.^4);

% Convert magnitude response to dB

InsertionLoss = -20*log10(max(H,eps));

figure;

plot(f/1e9,20*log10(max(H,eps)),'k','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Microwave Bandpass Filter Response');

yline(-3,'--r','-3 dB');

%% Objective 4: Filter Performance Evaluation

Passband = f(H >= 1/sqrt(2));

if ~isempty(Passband)

f_pass_low = min(Passband);

f_pass_high = max(Passband);

BW_3dB = f_pass_high-f_pass_low;

else

f_pass_low = NaN;

f_pass_high = NaN;

BW_3dB = NaN;

end

Stopband = f(H < 0.1);      % Below -20 dB magnitude

if ~isempty(Stopband)

StopbandStart = min(Stopband);

else

StopbandStart = NaN;

end

fprintf('\nFILTER PERFORMANCE\n');

fprintf('Center Frequency = %.3f GHz\n',fCenter/1e9);

fprintf('Specified Bandwidth = %.3f GHz\n',BW/1e9);

fprintf('Lower Passband Edge = %.3f GHz\n',f_pass_low/1e9);

fprintf('Upper Passband Edge = %.3f GHz\n',f_pass_high/1e9);

fprintf('Measured 3-dB Bandwidth = %.3f GHz\n',BW_3dB/1e9);

fprintf('Minimum Insertion Loss = %.2f dB\n',min(InsertionLoss));

fprintf('Stopband threshold = -20 dB\n');

fprintf('First frequency below -20 dB = %.3f GHz\n',StopbandStart/1e9);

Expected Output:

| Parameter | Expected values |
| --- | --- |
| Z0 | 50 |
| ZL | 30+j20 |
| Initial return loss | 7.66 dB |
| Matched return loss | Very High |
| Filter frequency | 2.366 GHz |
| Filter bandwidth | 0.8GHz |
| Filter insertion loss | 0dB |

Output:

INITIAL LOAD IMPEDANCE = 30.00 + j20.00 ohms

INITIAL RETURN LOSS = 9.29 dB

IDEAL MATCHED RETURN LOSS = 3.52 dB

OPTIMUM FREQUENCY = 2.401 GHz

RETURN LOSS AT OPTIMUM = 30.46 dB

Result:

The impedance-matching network and microwave bandpass filter were investigated using MATLAB. The reflection characteristics, return loss, filter passband, stopband, insertion loss and bandwidth were evaluated using simulated frequency responses.

FILTER PERFORMANCE

Center Frequency = 2.366 GHz

Specified Bandwidth = 0.800 GHz

Lower Passband Edge = 2.002 GHz

Upper Passband Edge = 2.797 GHz

Measured 3-dB Bandwidth = 0.795 GHz

Minimum Insertion Loss = 0.00 dB

Stopband threshold = -20 dB

First frequency below -20 dB = 1.000 GHz

Plots: